DropCeiling: Empowering Ubiquitous Objects with Adaptive Intelligence

Abstract

DropCeiling is an open-source system that turns standard LED office light panels into an intelligent agent, driving an Augmented Reality character whose position, brightness, and falloff respond to people on the sidewalk. Two cameras, calibrated with ARuco markers, track pedestrians and split the space into an active zone and a passing-traffic zone. Interaction is entirely implicit: presence alone is the input, with no screen, application, or instruction.

The character’s behavior arises from three nested control loops. The fastest reacts to who is present; a slower loop reads short-term trends to anticipate; the slowest, across hours and days, adjusts twelve meta-parameters that form the character’s evolving personality. An interaction budget meters how quickly that personality can change, keeping the system from overreacting to noise or settling into routine, so it remains adaptive across a sustained run rather than getting complacent. Every frame, the whole apparatus resolves to one virtual point light, from which each panel brightens by distance.

Deployed continuously for fifty-four days, the installation took in roughly one million pedestrian interactions and measurably evolved over time. It was authored through a human-AI process in which a three-dimensional spatial model served as the shared reference, becoming the installation’s brain.